DATE : 16-SEP-24

RIG:       ADM-688

WELL: Zulf-1261

Mixing fresh 83 PCF Barite Free Rheliant OBM

Pit #6 & pit #7

- Mix fresh mud in each pit by the same sequence as the following:

| CHEMICALS | QUANTITY |  | NOTES |
| --- | --- | --- | --- |
| Safra Oil | 350 | BBL |  |
| Sure Mul | 10 | drum |  |
| Sure Wet | 4 | drum |  |
| Lime | 60 |  |  |
| VG supreme | 15 |  | Use a small hose of water while mixing |
| Rhethick | 1 |  |  |
| VG plus | 15 |  | Use a small hose of water while mixing |
| Ecotrol | 24 |  |  |
| Brine (CaCl2) | 60 |  |  |
| Sea water | 20 | BBL |  |
| Keep all agitators and hopper working for 30 min |  |  |  |
| Marble fine | 45 | Big bag | Final M.WT= 83 PCF |

Thanks, Work safe

For any question call mud engineer

------------------------------------------------------------------------------------------------------------------

CLEAN OUT CEMENT & DV

Pit#2: Sea water

- Fill up pit# 2 with sea water, active pit while CLEAN OUT CEMENT & DV (all return over board)

------------------------------------------------------------------------------------------------------------------

Displace to 70 PCF WBM, TOC @ +/- 4670 FT.

Before Displacement:

- Pit#2:

  - Fill up pit#2 = 300 BBL Sea water.

  - Transfer from pit#6 to pit #2  150 BBL of 82 PCF WBM.

  - Pit #2 : Final volume = 450 BBL , M.WT = 70 PCF.

- Pit#6:

  - Transfer from pit#6 to Slug pit #10  80  BBL of 82 PCF WBM.

  - Fill up pit#6 = 270 BBL Sea water.

  - Pit #6: Final volume = 490  BBL , M.WT = 70 PCF.

------------------------------------------------------------------------------------------------------------------------------------------------------------

Displace to 70 PCF WBM, TOC @ +/- 4670 FT.

- Keep sand trap isolated (short system)

- Transfer line from pit # 6 to pit # 2 READY

- Mud pits ditch line return gate are open to pit # 2.

- Displacement Procedures:

  - Slug # 9    40 BBL Hi-Vis

  - Pit # 2      Active for 70 PCF WBM

  - Pit#6        top up WBM from (pit#6 ) to Active pit (Pit#2)

  - Sea water return over board

  - Once get WBM on surface, stop dump and take return (short system) to Pit # 2