DATE : 28-SEP-24

RIG:       ADM-688

WELL: Zulf-1262

MIXING PLAN

83 PCF Barite Free Rheliant OBM

# Objective:

Mix fresh barite free oil-based mud (OBM) in each pit according to the specified sequence to achieve a final mud weight of 83 PCF.

# Procedure:

Pit #6 & pit #7:

For both Pit #6 and Pit #7, add the following chemicals in the specified sequence, ensuring thorough mixing of each component before proceeding to the next.

| NO. | Chemical | Quantity | Notes |
| --- | --- | --- | --- |
| 1 | Safra Oil | 330 BBL |  |
| 2 | Sure Mul | 10 drums |  |
| 3 | Sure Wet | 4 drums |  |
| 4 | Lime | 60 sacks |  |
| 5 | VG Supreme | 15 sacks | Mix Slowly, 3 min per sack (use a small hose of water while mixing) |
| 6 | Rhethick | 1 drum |  |
| 7 | VG Plus | 15 sacks | Mix Slowly, 3 min per sack (use a small hose of water while mixing) |
| 8 | Ecotrol | 24 sacks |  |
| 9 | Brine (CaCl2) | 60 BBL | Transfer 60 BBL of brine from Brine tank |
| 10 | Sea Water | 20 BBL | Add 20 BBL of sea water |
| 11 | Keep the hopper and agitators running for a minimum of 30 minutes |  |  |
| 12 | Marble Fine | 45 big bags | Final M. WT = 83 PCF |

# 4. Safety Precautions:

- Follow all safety protocols during the mixing process.

- Ensure all personnel involved wear appropriate PPE (Personal Protective Equipment).

- In case of any issues or uncertainty, contact the mud engineer immediately.